# Rig PM Pro: Sourcing Gaps Check

Research output, checked 2026-10-17. Source: the earlier report Rig-PM-Pro-Manufacturer-PM-Research-2026-10-17.docx, section 4 gap list. Nothing here is in the PM database or approved. Quality Assurance verifies each entry and Joe signs off. Conflicts are flagged, not resolved.

# 1. Summary

Closed with official sources:

Freightliner Cascadia chassis intervals, from two DTNA public manuals on the DTNA content host (not the login portal). Model years are not stated in either.

Volvo 2022 GHG and newer poster compared with the 2017-2020 poster. Differences listed in section 3.

Thermo King APU (TriPac Envidia 2) service table and Thermo King truck reefer (T-Series) service table. Model years not stated in either.

Carrier Transicold truck reefer (Supra 560, 660, 760, 860, 960) service schedule. Revision Rev D, April 2020.

Partly closed:

Peterbilt chassis: a PACCAR 2017 maintenance manual was found, but it is a generic 'Truck Model Example' with coded interval columns and no model coverage. Not usable for the 379 or 579 yet.

International A26 engine: the service-reminder document was found, but it gives no oil interval. It points to the engine manual, which was not found.

Still open:

Kenworth T680 and W900 chassis: three Kenworth operator's manuals were refused by the fetch tool under robots rules. No maintenance table was read.

Peterbilt 379 and 579 chassis-specific intervals. The 579 operator's manual has no interval table.

International LT and LoneStar chassis: not found.

International A26 oil interval: the engine manual it refers to was not found.

Thermo King TriPac 3 and TriPac Evolution APU manuals: found in search, not read.

Carrier Vector: found in search, not read. It is a trailer unit, so it is out of scope for truck reefers.

Carrier 20X and 35X mileage points: still unconfirmed from the earlier report. Not rechecked here.

Volvo model year 2021 (no poster covers it). Carried over.

Model years for every Cascadia, Thermo King and Carrier item. QA to confirm.

# 2. Sourced intervals by make

## 2.1 Freightliner Cascadia chassis (DTNA)

Two DTNA manuals. Both have the same Schedule I and II figures. Each lists a different model coverage. Intervals are mileage-based, with a few hour and month footnotes.

| Item | Interval | Model coverage | Source and page | Date checked |
| --- | --- | --- | --- | --- |
| M1 (maintenance number), Schedule I (under 60,000 mi a year) | Every 10,000 mi | PT126SLP, PT126DC, PE116SLP, PE116DC (STI-500-10) | New Cascadia Maintenance Manual STI-500-10, Table 1, p. 00/2 | 2026-10-17 |
| M2, Schedule I | Every 50,000 mi | Same as above | STI-500-10, Table 1, p. 00/2 | 2026-10-17 |
| M3, Schedule I | Every 150,000 mi | Same as above | STI-500-10, Table 1, p. 00/2 | 2026-10-17 |
| Initial maintenance (IM), Schedule I | First 10,000 mi | Same as above | STI-500-10, Table 1, p. 00/2 | 2026-10-17 |
| Schedule II (over 60,000 mi a year): IM / M1 / M2 / M3 | First 25,000 / every 25,000 / every 100,000 / every 300,000 mi | Same as above | STI-500-10, Table 1, p. 00/2 | 2026-10-17 |
| Versajust slack adjuster inspection and lubrication (MOP 42-11) | Every 25,000 mi, 3 months, or 500 operating hours, whichever first | Same as above | STI-500-10, pp. 00/9, 00/13 | 2026-10-17 |
| Automatic drain valve (MOP 42-12) | Every 6 months or 1,500 operating hours, whichever first | Same as above | STI-500-10, pp. 00/9, 00/13 | 2026-10-17 |
| Clutch release bearing, Eaton Fuller UltraShift PLUS (MOP 25-01) | Every 50,000 mi or 3 months | Same as above | STI-500-10, pp. 00/9, 00/12 | 2026-10-17 |
| Air dryer desiccant, Bendix oil-coalescing cartridge (MOP 42-02) | Once a year, regardless of mileage | Same as above | STI-500-10, pp. 00/9, 00/12 | 2026-10-17 |
| Detroit axle knuckle pin and tie rod lubrication (MOP 33-01, 33-03), Schedule I | Once a year or every 25,000 mi, whichever first | Same as above | STI-500-10, pp. 00/10, 00/12 | 2026-10-17 |
| Eaton Fuller transmission fluid, synthetic | Extended to 500,000 mi | Same as above | STI-500-10, pp. 00/11, 00/12 | 2026-10-17 |
| Same schedule figures as above, plus CNG fuel cylinder (47-08) | CNG cylinder: every year or 100,000 mi, whichever first | CA113DC, CA113SLP, CA125DC, CA125SLP. No model year stated. | Cascadia Maintenance Manual STI-478-6, Table 1 and p. 00/11 | 2026-10-17 |

Not covered: the Detroit DD13 and DD15 engine intervals are a separate item (see the earlier report). The detailed M1, M2 and M3 operation lists (Tables 4 to 8) were not transcribed. Only the schedule figures and footnotes were read.

## 2.2 Peterbilt chassis (PACCAR)

PACCAR document Y53-6077-1B1, revision (03/17), copyright 2017. Its cover reads 'Truck Model Example'. No model years or model coverage are stated. Many entries use coded interval columns (I, A to E) that the excerpt does not define, so those are not listed. https://www.peterbilt.com/static-assets/documents/resources/conventional_model_maintenance_manual.pdf

| Item | Interval | Model coverage | Source and page | Date checked |
| --- | --- | --- | --- | --- |
| Air dryer overhaul, on-highway | 360,000 mi | Not stated (generic) | Y53-6077-1B1, p. 5-39. Conflicts with p. 5-61 (see section 3). | 2026-10-17 |
| Air system moisture check | Every 900 operating hours, 25,000 mi, or 3 months | Not stated | p. 5-60 | 2026-10-17 |
| Main and auxiliary transmission drain and flush, on-highway | 500,000 mi | Not stated | p. 5-26 | 2026-10-17 |
| LMS hubs (Dana), synthetic lubricant / mineral lubricant | 500,000 mi / 350,000 mi (may vary with inspection) | Not stated | p. 5-24 | 2026-10-17 |
| Spicer driveshaft U-joints and slip members (SPL140, 140HD, 170, 170HD, 250, 250HD), off-highway | First 350,000 mi, then every 100,000 mi | Not stated | p. 5-34 | 2026-10-17 |
| Engine mounts inspection | Every 60,000 mi | Not stated | p. 5-13 | 2026-10-17 |
| Exhaust system and cab ventilation inspection | Every 15,000 mi | Not stated | p. 5-42 | 2026-10-17 |
| Meritor axle lubricant change | 120,000 to 500,000 mi, depending on lubricant and use | Not stated | p. 5-53 (chart) | 2026-10-17 |
| Eaton and Dana axle lubricant change | 60,000 to 350,000 mi, depending on lubricant and use | Not stated | p. 5-54 (chart) | 2026-10-17 |
| Three-point safety belt inspection | Every 20,000 mi | Not stated | pp. 5-40, 5-78 | 2026-10-17 |
| Tire pressure check | Weekly, cold, with calibrated gauge | Not stated | p. 5-29 | 2026-10-17 |

Peterbilt 579 operator's manual (Y53-6110-1A1, revision (01/21), copyright 2021, cover 'MODEL 579'):

No maintenance interval table. Daily and weekly checks only, with no mileage, hour or month intervals. One interval found: the optional vocational 20k front axle kingpin joint and tie rod ends, 'every 50 hours', p. 37. Not a PM source for the 579. https://www.peterbilt.com/static-assets/documents/resources/2021_new_model_579_operators_manual.pdf. Checked 2026-10-17.

## 2.3 Kenworth chassis (PACCAR)

Not sourced. Three official Kenworth operator's manuals were identified: Y53-1200-1l1 (T680, T880, W990), Y53-1203-1e1 (W900, T660, T800, C500, 963) and Y53-1334-1c1. The fetch tool refused all three under robots rules, and no other route was tried. Status: open. Joe could download one of these and send it for reading.

## 2.4 Volvo: 2022 GHG and newer poster compared with 2017 to 2020 poster

2022 poster: PV960-V-SaaG-011023, 'Manufactured Year 2022 GHG and Newer', VNL, VNR, VNX, VHD, VAH (no date shown). 2017-2020 poster: VNL, VNR, VNX, VHD, VAH 'Manufactured Year 2017-2020' (no date or revision shown). Intervals are whichever occurs first. Duty bands: Economy above 7.5 mpg, Normal above 6, Heavy above 5, Severe below 5. The 2017-2020 poster labels only Normal, Heavy and Severe.

| Item | 2022 GHG and newer poster | 2017 to 2020 poster | Difference |
| --- | --- | --- | --- |
| Engine oil spec | Volvo VDS-5 | Volvo VDS-4.5 | Spec changed |
| Engine oil and filters, Normal (D11/D13) | 60,000 mi / 95,000 km / 1,700 h (Premium oil) | 60,000 mi / 95,000 km / 1,700 h | Same |
| Engine oil and filters, Heavy | 45,000 mi / 70,000 km / 1,400 h | 45,000 mi / 70,000 km / 1,300 h | Hours differ (1,400 vs 1,300) |
| Engine oil and filters, Severe | 35,000 mi / 55,000 km / 825 h | 35,000 mi / 55,000 km / 825 h | Same |
| Engine oil and filters, Economy | 75,000 mi / 120,000 km / 2,000 h | Not on poster | 2022 only |
| Oil, other-than-Premium oil values | Normal 55,000 mi / Heavy 40,000 mi / Severe 30,000 mi | Normal 55,000 mi / Heavy 40,000 mi / Severe 30,000 mi | Miles same; Heavy hours 1,300 (2022) vs 1,200 (2017-2020) |
| Idle footnote | If idle time over 30%, use next lower drain interval | Same footnote | Same |
| Extended-life coolant | 750,000 mi / 1,200,000 km / 96 months | 750,000 mi / 1,200,000 km / 96 months | Same |
| Coolant filter | 150,000 mi / 240,000 km / 12 months | 150,000 mi / 240,000 km / 12 months | Same |
| Valve adjustment | 150,000 mi, then every 300,000 mi; 12 months, then every 24 months | Same | Same |
| Fan and accessory drive belts (replace) | 450,000 / 360,000 / 315,000 / 210,000 mi; 36 and 12 months (tiers unlabeled) | 300,000 and 150,000 mi (unlabeled); 36 and 12 months | Differ. Tier mapping unclear in both. |
| Air filter and fuel tank vent filter | Inspect at each oil drain; replace at max restriction; 24 and 12 months | Replace at max restriction; 12 and 6 months | Time values differ. Mapping unclear. |
| Diesel particulate filter (clean), Normal | 480,000 mi / 772,000 km / 12,000 h | 400,000 mi / 650,000 km / 10,000 h (unlabeled columns; second column 250,000 mi / 150,000 km / 4,500 h) | Differ. 2017-2020 mapping unclear. Earlier report flagged its km values as possible typos. |
| DEF pump main filter and tank neck filter, Normal | 240,000 mi / 386,000 km / 7,000 h | 150,000 mi / 240,000 km / 4,500 h (no duty split) | Differ |
| Aftertreatment hydrocarbon injector | Not on poster | 150,000 mi / 240,000 km / 4,500 h | 2017-2020 only |
| I-Shift transmission oil and filter, Normal | 500,000 mi / 800,000 km / 60 months | 500,000 mi / 800,000 km / 60 months (first column) | Same figures. 2017-2020 columns unlabeled. |
| Power steering fluid and filter | 150,000 mi / 240,000 km / 12 months | Same | Same |
| Air dryer coalescing cartridge | 150,000 mi / 240,000 km / 12 months | Same | Same |
| Chassis lubrication | 15,000 / 10,000 mi (tier unclear) | Same | Same. Tier mapping unclear in both. |
| Electrical connectors check | Every 6 months | Every 6 months | Same |
| Three-axle alignment | Every 12 months (minimum) | Not listed | 2022 only |

## 2.5 Thermo King APU: TriPac Envidia 2

Operator's Manual TK 57250-19-OP-EN, Revision B, dated 01 Mar 2024, supersedes June 2023. Maintenance tables on pp. 40 to 45. The manual gives no model years. It refers to the TriPac 3 maintenance manual, TK 57117-19-MM. The table columns were inferred from the text layout, so QA should check them against the printed page. https://www.thermoking.com/content/dam/thermoking/documents/products/TriPac-Envidia-Operators-Manual.pdf

| Item | Interval | Model coverage | Source and page | Date checked |
| --- | --- | --- | --- | --- |
| Engine oil and filter change | 2,000 h with Thermo King filter and CK-4 or better oil; 500 h with other filter and CI-4 or better oil | TriPac Envidia 2 | pp. 40-41 | 2026-10-17 |
| Check engine oil level, coolant level, belts, unusual noise | Pretrip, every 500 h, annual (2,000 h) | TriPac Envidia 2 | p. 40 | 2026-10-17 |
| Adjust engine valves | 1,500 h | TriPac Envidia 2 | p. 40 | 2026-10-17 |
| Test fuel injection nozzles | At least every 3,000 h | TriPac Envidia 2 | p. 40 | 2026-10-17 |
| Replace fuel return lines | Every 10,000 h or sooner | TriPac Envidia 2 | p. 40 | 2026-10-17 |
| Anti-freeze protection and coolant change | Every 2 years, or with truck coolant | TriPac Envidia 2 | p. 40 | 2026-10-17 |
| ELC coolant (closed loop, red ELC) | Every 5 years or 12,000 h | TriPac Envidia 2 | p. 40 | 2026-10-17 |
| Tractor alternator, battery terminals, battery load test, battery fuses, Envidia 2 electrical | Semi-annual (1,000 h) and annual (2,000 h) | TriPac Envidia 2 | pp. 42-43 | 2026-10-17 |
| Evaporator return air filter; evaporator air inlet, outlets, drain valves | Pretrip, semi-annual, annual (2,000 h) | TriPac Envidia 2 | pp. 43 | 2026-10-17 |
| Condenser coil debris check | Semi-annual and annual | TriPac Envidia 2 | p. 44 | 2026-10-17 |
| Diesel fuel-fired heater (optional): run | At least 20 minutes each month | Optional heater | p. 44 | 2026-10-17 |

## 2.6 Thermo King truck reefer: T-Series self-powered truck units

Operator Manual TK 60483-OP, Rev. 14, dated 03-19 (copyright 2018). Maintenance table on p. 52. Covers self-powered trucks only, not trailers or APUs. Models: T-1200R, T-1200R Intermodal, T-1000R, T-800R, T-600R, T-560R, and SPECTRUM variants of T-1200R, T-1000R and T-800R. No model years stated. https://www.emea-user-manuals.thermoking.com/content/dam/tk-emea-usermanuals/pdf/Truck%20Self%20Powered/Operators/TK%2060483-1-OP%20T-Series%20Operator%20Manual%20Rev.%2014%2003-19_EN.pdf

| Item | Interval | Model coverage | Source and page | Date checked |
| --- | --- | --- | --- | --- |
| Interim inspection | Every 1,000 h or 6 months, whichever first | T-Series (all listed models) | p. 52 | 2026-10-17 |
| Complete PM | Every 2,000 h or 24 months, whichever first | T-Series (all listed models) | p. 52 | 2026-10-17 |
| Full service (A, B, C) | Every 4,000 h or 48 months, whichever first. Mapping of A, B, C to intervals not clear in text. | T-Series (all listed models) | p. 52 | 2026-10-17 |
| Full service, T-560R exception | Every 1,000 h, at an authorized Thermo King dealer | T-560R | p. 53 | 2026-10-17 |
| Pretrip inspection | Each pretrip (no interval) | T-Series | p. 52 | 2026-10-17 |

## 2.7 Carrier Transicold truck reefer: Supra 560, 660, 760, 860, 960

Operator's Manual Document 62-11858, Rev D, revision date 4/21/2020. Truck unit with Cab Command controller (not trailer). Unit maintenance schedule on pp. 31 to 32. Presented as bulleted lists, not a table. https://locator.ttdealers.carrier.com/upload/62-11858_D.pdf

| Item | Interval | Model coverage | Source and page | Date checked |
| --- | --- | --- | --- | --- |
| Lube oil and filter replacement | Every 1,500 engine hours | Supra 560, 660, 760, 860, 960 | p. 31 | 2026-10-17 |
| Oil drain and filter replacement, time limit | Maximum once a year (every 12 months) | All Supra models | p. 32 | 2026-10-17 |
| Fuel filter; air filter cartridge; belt tension; battery terminals and fluid | Every 1,500 engine hours | All Supra models | p. 31 | 2026-10-17 |
| Compressor oil level; rocker arms; bearings; refrigerant level; coil cleaning | Every 1,500 engine hours | All Supra models | p. 31 | 2026-10-17 |
| Anti-freeze change and coolant flush | Every 12,000 engine hours | All Supra models | p. 32 | 2026-10-17 |
| Coolant replacement, time limit | Every 12,000 hours or 5 years | All Supra models (units ship with Extended Life Coolant) | p. 32 | 2026-10-17 |
| Clean and adjust fuel injectors | Every 12,000 engine hours | All Supra models | p. 32 | 2026-10-17 |

## 2.8 International A26 engine (service interval document)

Document A26_SI_03172022, 'Change Oil Service Interval', copyright 2022 Navistar. Pages 1 to 5. https://www.international.com/-/media/Project/TratonGroup/International/International/Vehicles/Resources/Tem-Body-Builder/Engine-Media/EPA-21-International-A26/A26_Service_Interval_PDF.pdf?rev=68969439a1c8401cb4cdce781a375e0e

Oil interval: not stated. The document sets the oil-change reminder counters but does not state an oil interval. It says to set them to the values in the engine manual, section 'MAINTENANCE SCHEDULE AND SERVICE PROCEDURES' (pp. 3, 5). That manual was not found.

Example only. Example B, p. 5: 25,000 miles for an over-the-road driver. This is an example in the document, not a manufacturer interval.

Flag. Allowed range for the distance counter is 0 to 1,334.384 mi (p. 3). This does not fit the 25,000 mi example. Possible typo or unit error. Verify against the controller.

# 3. Conflicts flagged (not resolved)

Volvo posters. Volvo 2022 and 2017-2020 posters differ on DPF hours (12,000 vs 10,000 at Normal), DEF filter (240,000 mi and 7,000 h vs 150,000 mi and 4,500 h), belts, air filter times, the hydrocarbon injector, and oil spec. The 2017-2020 DPF and belt columns are unlabeled. QA to confirm the model year that applies to each figure.

PACCAR air dryer. Peterbilt PACCAR 2017 manual: air dryer overhaul 360,000 mi on p. 5-39, but 350,000 mi or 10,800 h, or 2 to 3 years, on p. 5-61.

Carrier oil change. Carrier Supra: oil change every 1,500 engine hours (p. 31) and maximum once a year (p. 32). The manual does not say which controls. QA to decide the rule before use.

Thermo King T-560R. Thermo King T-Series: general schedule is 1,000 h, 2,000 h and 4,000 h. T-560R requires full service every 1,000 h (p. 53). Applies to the T-560R only, per the document.

Thermo King oil. Thermo King TriPac Envidia 2: the engine oil interval depends on the filter used (2,000 h or 500 h). Not a conflict, but QA must confirm which filter and oil the unit uses.

Freightliner. Freightliner: two DTNA manuals have identical schedule figures but different model coverage (PT and PE in STI-500-10; CA113 and CA125 in STI-478-6). The STI-500-10 cover is dated 02/03/2022, but its schedule pages are dated October 2020 and its running headers show 2017 to 2020 revisions. QA to confirm which document applies to which Cascadia.

Layout inference. The Thermo King TriPac Envidia 2 table columns and the DTNA footnote tables were read from text layout. QA should check the printed pages.

# 4. Not reached, and out of scope

Kenworth operator's manuals Y53-1200-1l1, Y53-1203-1e1 and Y53-1334-1c1: fetch tool refused under robots rules.

International A26 service interval document redirected (302) to an address the fetch tool did not follow. The International LT and bodybuilder service manual s082852aa.pdf were not read.

ZeptoMail getting-started page: permission refused by the fetch tool (Zoho check only).

Carrier T-345 generator set service page was read and excluded. It is not a reefer.

The DTNA login portal (DTNAportal.com) was not used. The public DTNA content host was used instead.

Carrier Vector 1550 (62-12113) and Thermo King TriPac 3 (TK 57075-19-OP-EN) appeared in search only and were not read.

# 5. Next steps

QA to confirm the model years for each Cascadia, Thermo King and Carrier item, and check the flagged layout inferences against the printed pages.

Joe to decide whether he can download the three Kenworth manuals and send them for reading, since the fetch tool was refused.

Next pass on the TriPac 3 and TriPac Evolution APU manuals, and the Carrier Vector and 20X/35X mileage points.

No entries go to the PM database until QA verifies them and Joe approves the database work.

# 6. Sources

| Document | URL | Status (checked 2026-10-17) |
| --- | --- | --- |
| DTNA New Cascadia Maintenance Manual STI-500-10 | https://dtnacontent-dtna.prd.freightliner.com/content/dam/public/dtna-servicelit/dtna/pdfs/en_us/freightliner/drivers-manuals/New%20Cascadia%20Maintenance%20Manual_.pdf | Read. Official Freightliner content host. |
| DTNA Cascadia Maintenance Manual STI-478-6 | https://dtnacontent-dtna.prd.freightliner.com/content/dam/public/dtna-servicelit/dtna/pdfs/en_us/freightliner/drivers-manuals/Cascadia%20Maintenance%20Manual_.pdf | Read. |
| PACCAR Y53-6077-1B1 (2017), Peterbilt host | https://www.peterbilt.com/static-assets/documents/resources/conventional_model_maintenance_manual.pdf | Read. Generic model, coded columns. |
| Peterbilt 579 operator's manual Y53-6110-1A1 (2021) | https://www.peterbilt.com/static-assets/documents/resources/2021_new_model_579_operators_manual.pdf | Read. No interval table. |
| Kenworth T680/T880/W990 operator's manual Y53-1200-1l1 | https://www.kenworth.com/media/rwofr2dy/y53-1200-1l1.pdf | Refused (robots rules). |
| Kenworth W900 operator's manual Y53-1203-1e1 | https://www.kenworth.com/media/2nklfejp/y53-1203-1e1_en.pdf | Refused (robots rules). |
| Kenworth operator's manual Y53-1334-1c1 | https://www.kenworth.com/media/pkdfkvdy/y53-1334-1c1.pdf | Refused (robots rules). |
| Volvo SAAG 2022 GHG and newer poster PV960-V-SaaG-011023 | https://www.volvotrucks.us/media/vtna/files/saag-2022-ghg-newer-poster-011023-eng.pdf | Read. |
| Volvo SAAG 2017-2020 poster | https://www.volvotrucks.us/media/vtna/files/shared/parts-and-services/maintenance-intervals/2021/saag-2017-2020-poster-062221-eng.pdf | Read. |
| Thermo King TriPac Envidia 2 TK 57250-19-OP-EN Rev B | https://www.thermoking.com/content/dam/thermoking/documents/products/TriPac-Envidia-Operators-Manual.pdf | Read. Columns inferred. |
| Thermo King T-Series TK 60483-OP Rev 14 | https://www.emea-user-manuals.thermoking.com/content/dam/tk-emea-usermanuals/pdf/Truck%20Self%20Powered/Operators/TK%2060483-1-OP%20T-Series%20Operator%20Manual%20Rev.%2014%2003-19_EN.pdf | Read. |
| Carrier Transicold Supra 560-960 operator's manual 62-11858 Rev D | https://locator.ttdealers.carrier.com/upload/62-11858_D.pdf | Read. |
| International A26 Change Oil Service Interval A26_SI_03172022 | https://www.international.com/-/media/Project/TratonGroup/International/International/Vehicles/Resources/Tem-Body-Builder/Engine-Media/EPA-21-International-A26/A26_Service_Interval_PDF.pdf?rev=68969439a1c8401cb4cdce781a375e0e | Read. No oil interval stated. |