# Rig PM Pro: Sourced Intervals, Batch 1

Research output, checked 2026-10-18. Priority list from Joe's plan of October 18, 2026. Each entry is a verified official source. Quality Assurance verifies each entry before it is final, and Joe signs off. Conflicts are flagged for Joe, not resolved. Source IDs (S1 to S7) are listed in Section 6.

# 1. Read this first: year ranges

None of the new sources states a model year range for the models it covers. The coordinator's rule is that a verified source replaces the user-entered interval for a make, model and year range. Without a stated year range, these sources can only be applied to make and model. Joe or QA needs to set the year range for each one before the label changes. Until then, the user-entered interval stays, with its label.

# 2. Status by priority

| Priority | Item | Status after batch 1 |
| --- | --- | --- |
| 1 | Freightliner Cascadia (chassis and Detroit DD13/DD15 engines) | Engine oil, fuel filter, coolant, belts, DEF filter and DPF verified (Section 3). Chassis intervals were reported earlier from DTNA manuals. Model years not stated for any Cascadia source. |
| 2 | Volvo VNL860 (Volvo posters cover the VNL family) | Verified for VNL, VNR, VNX, VHD and VAH by three posters (2017-2020, 2021, 2022). VNL860 is not named in any poster, so it is an assumption that it falls under the VNL family. The 2021 poster closes the model-year 2021 gap. |
| 3 | Peterbilt 579 chassis | OPEN. The 2024 operator's manual (Y53-6110-1B1) has its maintenance chapter outside the text the fetch tool could read. Direct download is blocked by the proxy. Engine MX-13 intervals from earlier PACCAR documents are not re-verified for the 579. |
| 4 | Thermo King S600 and C600 reefers | PARTIAL. Both are trailer units (Precedent S-600 and C-600), not truck units. Verified: extended-life coolant 5 years or 12,000 hours; EMI 3000 package extends standard intervals to 3,000 hours or 2 years. Full maintenance schedule not reached. |
| 5 | Carrier units | Vector 1550 verified (trailer). Vector 8100 partial (trailer, no hour or mile figures). Supra 560-960 (truck) was verified earlier and is not repeated here. |

# 3. New verified entries

Model years are not stated in any of these sources. Unit is shown with each interval. Items with 'or' mean whichever occurs first, per the source's footnote.

## 3.1 Detroit DD15 (Cascadia engine option, Gen 5 Operator Manual and 2021 brochure)

| Make and model (years) | Service | Interval and unit | Source, page | Conflict or flag |
| --- | --- | --- | --- | --- |
| DD15, all years stated | Engine oil and filter, Efficient Long Haul (above 7 mpg) | 75,000 mi | S1 p. 60; S2 p. 12 | Matches earlier Detroit page. Duty definitions differ (see section 4). |
| DD15 | Engine oil and filter, Long Haul (6.0-6.9 mpg) | 60,000 mi | S1 p. 60; S2 p. 12 | Same as above. |
| DD15 | Engine oil and filter, Short Haul (5.1-5.9 mpg) | 45,000 mi, or 1,000 h | S1 p. 60; S2 p. 12 | Brochure also lists 1 year. |
| DD15 | Engine oil and filter, Severe (under 5 mpg) | 35,000 mi, or 750 h | S1 p. 60; S2 p. 12 | Brochure also lists 6 months. |
| DD15 | Oil filter and engine fuel filter | At every oil change | S1 p. 60 | None. |
| DD15 | Frame-mounted fuel filter, ELH and LH / SH / Severe | 100,000 mi / 90,000 mi or 1,000 h / 70,000 mi or 750 h | S1 p. 60 | Short haul differs from DD13 (80,000 mi). Engine-specific. |
| DD15 | Coolant, standard life: maintain / replace | Maintain at every oil change or 1 year (6 months Severe); replace 300,000 mi | S1 p. 60 | None. |
| DD15 | Coolant, extended life: replace | 600,000 mi (maintain every other oil change or 1 year) | S1 p. 60 | None. |
| DD15 | Belts (inspect at oil change) | Replace 200,000 mi (ELH and LH); 100,000 mi or 2,600 h (SH and Severe) | S1 pp. 60-61 | None. |
| DD15 | Diesel particulate filter | ELH over 640,000 mi or 14,000 h; LH 550,000-640,000 mi or 12,000-14,000 h; SH 455,000-550,000 mi or 10,000-12,000 h; Severe under 455,000 mi or 10,000 h | S1 p. 61 | Conflicts with earlier report (DD15 SH 11,000-11,500 h). SH cell garbled in text. |
| DD15 | DEF pump filter (all categories) | 500,000 mi or 3 years | S1 p. 61 | Matches earlier Detroit page (3 years or 500,000 mi). |
| DD15 | Valve lash adjustment | ELH 500,000 mi; SH 200,000 mi | S1 p. 60 | LH and Severe values not assigned in text. |
| DD13, all years stated | Engine oil and filter (ELH / LH / SH / Severe) | 65,000 / 55,000 / 40,000 mi or 1,000 h / 35,000 mi or 750 h | S1 pp. 58-59; S2 p. 12 | Matches earlier Detroit page. Duty definitions differ (see section 4). |
| DD13 | Frame-mounted fuel filter | 100,000 / 100,000 / 80,000 mi or 1,000 h / 70,000 mi or 750 h | S1 p. 59 | Engine-specific difference from DD15. |
| DD13 | Belts | Replace 150,000 mi (ELH and LH); 105,000 mi or 2,690 h (SH and Severe) | S1 p. 59 | None. |
| DD13 | Coolant replace (standard / extended) | 300,000 mi / 600,000 mi | S1 p. 59 | None. |
| DD13, DD15, DD16 on non-DFS oils (CJ-4 and similar) | Engine oil and filter | Long Haul 50,000 mi or 1,280 h; SH 35,000 mi or 895 h or 1 year; Severe 25,000 mi or 640 h or 6 months | S2 p. 13 (Table 5) | Reduced interval for non-DFS oils. Applies only when the oil is not DFS 93K222/93K223. |

DD13 DPF, DEF filter numeric figure, and the DD13 valve-lash mapping were not readable in the text and are not entered.

## 3.2 Volvo VNL family (2021 GHG and newer poster; VNL860 not named)

| Make and model (years) | Service | Interval and unit | Source, page | Conflict or flag |
| --- | --- | --- | --- | --- |
| VNL, VNR, VNX, VHD, VAH, 2021 GHG and newer (VNL860 not named) | Engine oil and filters, D11/D13, Premium VDS-5 oil: Normal / Heavy / Severe | Normal 60,000 mi / 95,000 km / 1,700 h; Heavy 45,000 mi / 70,000 km / 1,300 h; Severe 35,000 mi / 55,000 km / 825 h | S3 (single poster, no page) | Heavy hours 1,300 here and in 2017-2020; 1,400 in 2022 (conflict, section 4). |
| Same | Engine oil, other approved VDS-5 oil | Normal 55,000 mi / 90,000 km / 1,600 h; Heavy 40,000 mi / 65,000 km / 1,200 h; Severe 30,000 mi / 50,000 km / 750 h | S3 | Heavy hours 1,200 here; 1,300 in 2022. |
| Same | Diesel particulate filter (clean) | 400,000 mi and 250,000 mi (unlabeled); 10,000 h and 4,500 h | S3 | Duty mapping unlabeled. Km pair (650,000 and 150,000) looks inconsistent and may be a transcription error. 2022 Normal is 480,000 mi. |
| Same | DEF pump main filter and tank neck filter | 150,000 mi / 240,000 km / 4,500 h | S3 | 2022 Normal is 240,000 mi / 7,000 h (conflict, section 4). |
| Same | Aftertreatment hydrocarbon injector | 150,000 mi / 240,000 km / 4,500 h | S3 | Not on 2022 poster. |
| Same | Coolant, extended life; coolant filter | 750,000 mi (96 months); filter 150,000 mi (12 months) | S3 | Same as 2017-2020 and 2022. |
| Same | Fan and accessory drive belts | 300,000 mi and 150,000 mi (unlabeled); 36 and 12 months | S3 | 2022 lists 450,000 / 360,000 / 315,000 / 210,000 mi. Tier mapping unclear in both. |
| Same | Air filter and fuel tank vent filter | At max restriction, or 12 months; at max restriction, or 6 months (unlabeled) | S3 | 2022 times differ (24 and 12 months). |
| Same | I-Shift transmission oil and filter | 500,000 mi / 800,000 km / 60 months (first column) | S3 | Second column 250,000 mi / 2,500 h (unlabeled). |
| Same | Power steering fluid and filter; air dryer cartridge | 150,000 mi / 240,000 km / 12 months each | S3 | Same as 2017-2020 and 2022. |

## 3.3 Thermo King Precedent S-600 and C-600 (trailer units, model years not stated)

| Make and model (years) | Service | Interval and unit | Source, page | Conflict or flag |
| --- | --- | --- | --- | --- |
| Precedent S-600, C-600, S-700 (TK 56218-2-OP-EN, Rev B, June 2021) | Extended-life coolant maintenance interval | 5 years or 12,000 hours | S4 pp. 22-23 (two reads differ) | Same as 2013 C-600 manual. |
| Precedent S-600, C-600, S-700 | EMI 3000 package extended standard intervals | 3,000 hours or 2 years, whichever first | S4 pp. 22-23 | Same as 2013 C-600 manual (S5 p. 23). |
| Precedent C-600 (TK-55692P-2-OP, May 2013 Rev 0, Portuguese) | Extended-life coolant, EMI 3000 | 5 years or 12,000 h; 3,000 h or 2 years | S5 pp. 22-23 (translated) | Consistent with S4. |
| S-600 and C-600 full maintenance schedule | Maintenance Inspection Schedule | NOT READ | S4 p. 141; S5 p. 205 | Open. Text windows did not reach the table. |

## 3.4 Carrier Vector 1550 and Vector 8100 (trailer units)

| Make and model (years) | Service | Interval and unit | Source, page | Conflict or flag |
| --- | --- | --- | --- | --- |
| Vector 1550 (62-12113, printed March 2019) | Engine oil change, petroleum oil | 3,000 hours or 2 years (maximum drain 2 years) | S6 p. 24 | Oil type sets the interval. Mobil Delvac 1 below. |
| Vector 1550 | Engine oil change, Mobil Delvac 1 (only approved synthetic) | 4,000 hours or 2 years (maximum drain 2 years) | S6 p. 24 | None. |
| Vector 1550 | Coolant drain, flush and refill (Extended Life Coolant) | 5 years or 12,000 hours | S6 p. 24 | None. |
| Vector 1550 | Fuel injectors; injection pump; engine valves | 1,500 h (footnote: EPA 40 CFR Part 89); 3,000 h; 4,000 h | S6 p. 23 | Injector and pump figures carry an EPA footnote. |
| Vector 1550 | Most other items | Service interval or yearly (no hour or mile figure) | S6 pp. 23-24 | 'Service interval' is not defined in hours or miles. |
| Vector 8100 (62-11784 Rev D, June 2021) | All maintenance items | No hour or mile figures. Only 'before loading' and 'every service interval or yearly' | S7 pp. 40-41 | No numeric interval for this unit. Not usable as a PM interval. |

## 3.5 Items carried from earlier batches (not new)

Freightliner Cascadia chassis (DTNA STI-500-10 and STI-478-6, pp. 00/2 to 00/14): intervals reported in the earlier sourcing report. Model years not stated, so no year range yet.

Carrier Supra 560, 660, 760, 860, 960 (truck, 62-11858 Rev D, April 2020): reported earlier. Its 1,500-hour oil change and once-a-year maximum conflict is still open.

# 4. Conflicts flagged for Joe

DD15 DPF hours. Detroit DD15 DPF, short haul: this batch gives 10,000-12,000 h (455,000-550,000 mi); the earlier report gave 11,000-11,500 h. The text for this cell is garbled, so QA should read the printed page 61 before choosing.

Duty definitions. Detroit duty bands: the Gen 5 manual defines Efficient Long Haul as over 60,000 mi a year and over 7 mpg, and Short Haul as 30,000 to 60,000 mi a year and 5.1 to 5.9 mpg. The brochure defines bands by mpg only. Which definition the app uses must be decided.

Davco override. Detroit DD15 footnote: 'Reaching the Davco change line supersedes these intervals' (p. 61). This is a rule that can override the table. Not modeled yet.

Volvo heavy hours. Volvo heavy-duty Premium oil hours: 1,400 (2022 poster) against 1,300 (2021 and 2017-2020 posters).

Volvo DEF filter. Volvo DEF pump filter, Normal: 240,000 mi / 7,000 h (2022) against 150,000 mi / 4,500 h (2021 and 2017-2020, no duty split).

Volvo DPF. Volvo DPF, Normal: 480,000 mi / 12,000 h (2022) against 400,000 mi / 10,000 h (2021 and 2017-2020, unlabeled columns). The 2021 km pair (650,000 and 150,000) looks inconsistent.

VNL860 assumption. Volvo posters give no model match for VNL860. Applying the VNL family figures to VNL860 is an assumption.

Carrier Vector units. Carrier Vector 8100 says 'service interval' without defining it. Vector 1550 uses hours for the same kind of item. The two manuals do not agree in format.

Unit type. Thermo King S-600 and C-600 are trailer units. The coordinator's list calls them reefers. Confirm they are the intended units before launch.

# 5. Still open after batch 1

Peterbilt 579 chassis: maintenance chapter of the 2024 operator's manual not read; the 2021 manual has no interval table; direct PDF download blocked by the proxy (403).

Peterbilt 579 engine (MX-13): earlier PACCAR E108 and E256 documents still conflict; not re-verified for the 579.

Thermo King S-600 and C-600 full maintenance schedule (S4 p. 141; S5 p. 205).

Cascadia model years for every source; Kenworth (robots rule); Carrier Vector 8100 hour intervals.

Volvo VNL860: no VNL860-specific source found. The VNL family posters are the only Volvo sources.

# 6. Sources

| ID | Document | URL | Checked |
| --- | --- | --- | --- |
| S1 | Detroit DD Platform Gen 5 Operator Manual, DDC-SVC-MAN-0217, revised 2025-08-26 (Section 9.03.01, pp. 58-61) | https://dtnacontent-dtna.prd.freightliner.com/content/dam/public/dtna-servicelit/ddc/pdfs/OperatorsManual/DDPlatform/DDC-SVC-MAN-0217_2026.pdf | 2026-10-17 |
| S2 | Detroit Lubricating Oil, Fuel, and Filters, DDC-SVC-BRO-0001 (copyright 2021; Tables 4 and 5, pp. 12-13) | https://dtnacontent-dtna.prd.freightliner.com/content/dam/public/dtna-servicelit/ddc/pdfs/Lube_Oil_Coolant/DDC-SVC-BRO-0001_2021.pdf | 2026-10-17 |
| S3 | Volvo SAAG poster, For VNL, VNR, VNX, VHD and VAH Manufactured Year 2021 GHG and Newer (no date or revision shown) | https://www.volvotrucks.us/media/vtna/files/shared/parts-and-services/maintenance-intervals/2021/saag-2021-ghg-newer-poster-062221-eng.pdf | 2026-10-17 |
| S4 | Thermo King Precedent Single Temperature Units C-600, S-600 and S-700 Operator's Manual, TK 56218-2-OP-EN, Rev B, June 2021 | https://www.thermoking.com/content/dam/thermoking/documents/technical-literature/TK-56218-1-OP-EN-Precedent-Single-Temperature-Units.pdf | 2026-10-17 |
| S5 | Thermo King Precedent C-600 Operator's Manual, TK-55692P-2-OP, May 2013, Rev 0 (Portuguese; translated) | https://www.thermoking.com/content/dam/thermoking/la/manuais/br/Precedent-C-600-Manual-Operacao-TK5592P.pdf | 2026-10-17 |
| S6 | Carrier Vector 1550 Trailer Refrigeration Units Operator Manual, 62-12113 (printed March 2019; Section 6.1, pp. 23-24) | https://locator.ttdealers.carrier.com/upload/62-12113.pdf | 2026-10-17 |
| S7 | Carrier Vector 8100 Trailer Refrigeration Unit Operator's Manual, 62-11784 Rev D, June 2021 (Section 24.1, pp. 40-41) | https://locator.ttdealers.carrier.com/upload/62-11784_D.pdf | 2026-10-17 |