# Rig PM Pro: Sourced Intervals, Batch 2 (Engines by Model)

Research output, checked 2026-10-18. Organized by make, then model, then engine, then year range, per Joe's change of October 18, 2026. Intervals are sourced per engine, not per make. Batch 1 is unchanged. Quality Assurance verifies each entry before it is final, and Joe signs off. Source IDs are in Section 6.

# 1. How to read this batch

Engine option. The engine option is verified when an official manufacturer page or document names that engine for that model.

Interval. The interval is verified when an official document gives an interval for that engine and the duty or application it applies to.

Status key. VERIFIED: option and interval both sourced. PARTIAL: one of the two sourced, or the interval covers part of the engine. CONFLICT: sources disagree, flagged for Joe. UNVERIFIED: no source, so the user-entered path covers it with a label.

Year ranges. None of the sources states a model year for the models listed. Year ranges are shown only where the source states a generation or launch date. A year range cannot be applied to the coordinator's replacement rule until Joe or QA sets it.

# 2. Summary by make and model

| Make and model | Engine options verified | Intervals verified per engine | Status |
| --- | --- | --- | --- |
| Volvo VNL 300, 400, 740, 760, 860 | D13 (current model) | D13 through the 2022 GHG and newer poster (family, D11/D13 pan) | VERIFIED for D13 |
| Volvo VNL 440, 640, 660, 840 | None | None | UNVERIFIED |
| Volvo D11, D16, Cummins X15, X12G on VNL | D11, X15, X12G named in 2021 brochure with no model mapping. D16 not found. | Family poster covers D11 and D13. X15 at engine level only. | UNVERIFIED for VNL models |
| Freightliner Cascadia (fifth generation) | DD13, DD15 at launch. Cummins X15 due 2025, X15N due 2026. DD16 not named. | DD13 and DD15 verified. X15 at engine level with a conflict. X15N and DD16 not verified for Cascadia. | PARTIAL |
| Peterbilt 579 | MX-13, MX-11, Cummins X15, X15N, ISX12N, L9N | MX-13 conflict (E108 vs E256). MX-11 oil only. X15 engine level with a conflict. Others not verified. | PARTIAL / CONFLICT |
| Kenworth T680, T880, W990 | MX-13 (T680, T880, W990). Cummins X15N (T680, T880). | MX-13 conflict (E108 vs E256). X15N not verified. | PARTIAL / CONFLICT |
| Mack Anthem | MP13 and MP13 HE (2026 model). MP7, MP8, MP8 HE and MP10 not mapped to Anthem. | Mack documents are family-level and do not name the engine. Not verified per engine. | UNVERIFIED for intervals |
| Mack Granite, Pinnacle, TerraPro | None verified | None verified | UNVERIFIED |
| International LT | S13 and Cummins X15 (current). A26 on LT from 2017 (not on the current page). | X15 at engine level with a conflict. S13 not verified. A26 has no oil interval. | PARTIAL / CONFLICT |
| International LoneStar, HX | None verified | None verified | UNVERIFIED |

# 3. Volvo VNL (all models, by engine)

Volvo's current VNL specification page lists five models, all with a D13 engine. Its product guide (dated January 2024) names only the D13. Earlier brochure (2021) also names D11, D13, D13TC, X12G and Cummins X15 for the VNL series without model mapping. No D16 was found in any VNL source.

## 3.1 VNL 300, 400, 740, 760, 860

| Engine | Year range (as stated) | Engine option source | Interval source and unit | Status |
| --- | --- | --- | --- | --- |
| Volvo D13 | Current VNL (guide dated 01/2024). No model year stated. | V1: D13 for VNL 300, 400, 740, 760 and 860 (405-455 hp; VNL 740, 760 and 860 also 500 hp). V2 p. 41: two new VNL ratings, 500 hp. | V4 (2022 GHG and newer poster), engine oil and filters, D11/D13 42L/44qt pan. Premium oil: Normal 60,000 mi / 95,000 km / 1,700 h; Heavy 45,000 mi / 70,000 km / 1,400 h; Severe 35,000 mi / 55,000 km / 825 h. Economy 75,000 mi / 120,000 km / 2,000 h. | VERIFIED |
| Volvo D13TC (turbo-compound) | Standard on VNL 740, 760, 860 in the 2021 brochure (p. 38). No model year stated. | V3 p. 24: D13TC 405-455 hp, 1,750-1,850 lb-ft. V3 spec table p. 70 gives 1,450-1,850 lb-ft (internal conflict). | No D13TC row in any poster. Poster rows cover D11/D13 generically. Not separately verified. | PARTIAL |

Note: the poster intervals cover the VNL family by name. Applying them to VNL 760 and 860 relies on the D13 option in V1. The 2022 poster has no VNL-model-specific rows.

## 3.2 VNL 440, 640, 660, 840

Not on Volvo's current specification page. No official source for these model names was read. Status: UNVERIFIED for every engine. User-entered path applies.

## 3.3 Engines named for VNL without model mapping

| Engine | Year range (as stated) | Engine option source | Interval source and unit | Status |
| --- | --- | --- | --- | --- |
| Volvo D11 | 2021 brochure (V3). No model year. | V3 p. 23: 385-425 hp, 1,450-1,550 lb-ft. No model mapping. | Family poster rows for D11/D13 (V4, V5, 2017-2020 poster, 2010-2016 poster). Not tied to a VNL model. | UNVERIFIED for models |
| Volvo D16 | None found | No D16 in any VNL source read. | None found | UNVERIFIED |
| Cummins X15 (diesel) | 2021 brochure (V3 p. 26): 400-565 hp. No model mapping. | V3 only. | Cummins X15 guides, EPA 2024 (C1) and EPA 2017 (C2). Engine level, not VNL-specific. Duty bands conflict (section 4). | PARTIAL |
| Cummins X12G (natural gas) | 2021 brochure (V3 p. 26): 400 hp. No model mapping. | V3 only. | None found | UNVERIFIED |

# 4. Freightliner Cascadia (fifth generation and earlier)

Freightliner's October 15, 2024 release (F1) names the DD13 and DD15 at launch, with a combined rating of up to 505 hp and 1,850 lb-ft. Cummins X15 is due in 2025 and X15N in 2026. Chassis intervals from the DTNA manuals (Batch 1 and the earlier report) apply regardless of engine. Model years are not stated.

| Engine | Year range (as stated) | Engine option source | Interval source and unit | Status |
| --- | --- | --- | --- | --- |
| Detroit DD13 | Fifth generation, available at launch (F1). Not split by year. | F1: DD13 available at launch; combined range up to 505 hp. | S1 pp. 58-59 (Gen 5 manual); S2 p. 12 (oil). Batch 1, section 3.1. | VERIFIED |
| Detroit DD15 | Fifth generation, available at launch (F1). | F1: DD15 available at launch; same combined range. | S1 pp. 60-61; S2 p. 12. DD15 short-haul DPF cell garbled in text. | VERIFIED |
| Detroit DD16 | Not stated for Cascadia. | F1 does not mention DD16. Not verified for Cascadia. | S2 p. 12: Long Haul 55,000 mi; Short Haul 40,000 mi, 1,000 h or 1 year; Severe 35,000 mi, 750 h or 6 months. Engine level only. | UNVERIFIED (option) |
| Cummins X15 (diesel) | Due 2025 (F1). Availability not yet confirmed. | F1: X15 due 2025, combined range 400-605 hp with X15N. | C1 (EPA 2024, Bulletin 6553662, Rev 11/24): oil drain Severe 25,000 mi / Short Haul 50,000 mi / Normal 60,000 mi / Light 75,000 mi, 12 months each; vocational 20,000 mi, 12 months or 500 h. C2 (EPA 2017, Bulletin 5410641, Rev 11/17): Severe 25,000 mi / Normal 35,000 mi / Light 50,000 mi, 500 h or 6 months. | PARTIAL / CONFLICT |
| Cummins X15N (natural gas) | Due 2026 (F1). | F1: X15N due 2026, same combined range. | Not verified. No natural gas guide read. | UNVERIFIED (interval) |
| Prior-generation Cascadia (no year) | Not stated. Detroit overview is dated about 2016 (F2). | F2: DD13 and DD15 each 400 hp. No Cascadia year or cab mapping. | S1 and S2 are Gen 5 documents. Prior-generation intervals not sourced. | UNVERIFIED (interval) |

# 5. Peterbilt 579, Kenworth T680 and W990, Mack Anthem, International LT

## 5.1 Peterbilt 579 (current model page, P1; no model years)

| Engine | Year range (as stated) | Engine option source | Interval source and unit | Status |
| --- | --- | --- | --- | --- |
| PACCAR MX-13, up to 510 hp | Not stated | P1 page. | E108 (rev 1/19/2015, MX-13 EPA 2010 only) and E256 (SB 1/6/2020, MX-13 and MX-11, oil and filter only). Conflict flagged in the earlier report. | CONFLICT |
| PACCAR MX-11, up to 445 hp | Not stated | P1 page. | E256 only: oil and filter. No aftertreatment interval. | PARTIAL |
| Cummins X15, up to 605 hp | Not stated | P1 page. | C1 (EPA 2024) and C2 (EPA 2017). Duty bands conflict. Year mapping not stated. | PARTIAL / CONFLICT |
| Cummins X15N, up to 500 hp | Not stated | P1 page. A Peterbilt press release (search result) names X15N natural gas on the 579, not fetched. | Not verified. | UNVERIFIED |
| Cummins ISX12N, up to 400 hp | Not stated | P1 page. | Not verified. | UNVERIFIED |
| Cummins L9N, 320 hp | Not stated | P1 page. | Not verified. | UNVERIFIED |
| 579EV (battery electric) | Not stated | P1 links a manual but gives no power rating. Excluded from the engine list. | Not verified. | UNVERIFIED |

## 5.2 Kenworth T680, T880, W990 (no model years)

| Engine | Year range (as stated) | Engine option source | Interval source and unit | Status |
| --- | --- | --- | --- | --- |
| PACCAR MX-13 CARB-compliant, 510 hp and 455 hp | Not stated. Release names T680, T880 and W990. | K1: names the MX-13 for T680, T880 and W990. Ratings 510 hp (1,850 lb-ft) and 455 hp (1,650 lb-ft). No release date stated. | E108 and E256 (as for Peterbilt 579). Conflict flagged. | CONFLICT |
| Cummins X15N (natural gas), 400-500 hp | Production start Q3 2024 (K2). T680 and T880 only, not W990. | K2: T680 and T880 specified with X15N. | Not verified. | UNVERIFIED (interval) |
| PACCAR MX-11 and Cummins X15 (diesel) | Not stated | Kenworth truck page refused by the fetch tool under robots rules. No option verified. | Not verified. | UNVERIFIED (option) |

## 5.3 Mack Anthem (2026 model)

| Engine | Year range (as stated) | Engine option source | Interval source and unit | Status |
| --- | --- | --- | --- | --- |
| Mack MP13, 415-515 hp | 2026 Anthem (M1, article dated 11/24/2025) | M1: MP13 and MP13 HE span 415-515 hp. M2: MP13 415-515 hp. | Mack SAAG 2022 GHG and newer (macktrucks.ca) and 2017-2020 schedules (earlier report). Documents are family-level and do not name the engine. | PARTIAL (option verified; interval not per engine) |
| Mack MP13 HE, 415-505 hp | 2026 Anthem (M1) | M1: MP13 HE within 415-515 hp range. M2: MP13 HE 415-505 hp. Ratings differ slightly. | Same as MP13: family-level only. | PARTIAL |
| Mack MP8, MP8 HE, MP7, MP10 | Not stated for Anthem | M2 lists MP8 415-505 hp, MP8 HE 415-445 hp, MP7 325-425 hp. MP10 not on M2. No Anthem mapping. | Not verified per engine. | UNVERIFIED |

## 5.4 Mack Granite, Pinnacle, TerraPro

No engine option was verified in this pass. Granite is on a legacy Mack page (not read). Status: UNVERIFIED for every engine. User-entered path applies.

## 5.5 International LT (current LT page, I1)

| Engine | Year range (as stated) | Engine option source | Interval source and unit | Status |
| --- | --- | --- | --- | --- |
| International S13, 12.74 L, 400-515 hp | Current LT (2024-25 brochure). No model year. | I1: S13 400-515 hp, 1,450-1,850 lb-ft. | No interval source found. The S13 document in search (S13 maximum standstill speed) is not a PM schedule. | PARTIAL (option only) |
| Cummins X15, 400-565 hp | Current LT | I1: X15 400-565 hp, 1,450-2,050 lb-ft. | C1 and C2 at engine level. Duty bands conflict (section 4). | PARTIAL / CONFLICT |
| International A26, 12.4 L, up to 475 hp | LT first shipments June 30, 2017 (I2). Not on the current LT page (I1). | I2: A26 in LT and RH from June 30, 2017. Current LT option not verified. | I3 (A26 Change Oil Service Interval, A26_SI_03172022): gives a reminder only. No oil interval. Engine manual it refers to not found. | PARTIAL (no interval) |
| LoneStar, HX | Not stated | Not on I1. No official source read. | Not verified. | UNVERIFIED |

# 6. Conflicts flagged for Joe (not resolved)

PACCAR MX-13. E108 (rev 1/19/2015, MX-13 EPA 2010 only) and E256 (SB 1/6/2020, MX-13 and MX-11) give different MX-13 oil intervals. Applies to Peterbilt 579 and Kenworth T680, T880, W990. Carried from the earlier report.

Cummins X15 duty bands. Cummins X15 EPA 2017 (C2) and EPA 2024 (C1) use different duty labels and different intervals. Normal 35,000 mi (C2) vs 60,000 mi (C1). Light 50,000 mi vs 75,000 mi. Severe 25,000 mi in both, but the bands differ (under 5.5 mpg vs 3-4.9 mpg). C2 also caps drain at 500 h and 6 months, while C1 uses 12 months. Applies to Cascadia, Peterbilt 579, Kenworth and International LT X15 options.

Detroit duty definitions. Detroit duty bands: the Gen 5 manual (S1) defines Efficient Long Haul by annual mileage and mpg. The brochure (S2) uses mpg only. Carried from Batch 1.

DD15 DPF. Detroit DD15 short-haul DPF (S1 p. 61) is garbled in the text. Carried from Batch 1.

Volvo heavy hours. Volvo D13 heavy oil hours: 1,400 (2022 poster) vs 1,300 (2021 and 2017-2020). Carried from Batch 1.

Volvo D13TC torque. Volvo D13TC torque: 1,750-1,850 lb-ft (2021 brochure p. 24) vs 1,450-1,850 lb-ft (spec table p. 70). Same brochure.

Volvo model list. Volvo VNL model list: the current spec page names VNL 300, 400, 740, 760, 860. A product guide summary named VNL 440, 640, 660, 840 as well. The second list is not confirmed by a source read in full.

Kenworth engine lists. Kenworth: the MX-13 release names T680, T880 and W990. The X15N release names T680 and T880 only. Both are official. Not a conflict, but the W990 engine list is narrower than the T680 list.

Mack MP13 HE rating. Mack MP13 HE: 415-505 hp (M2) vs 415-515 hp combined range (M1). Minor rating difference.

International A26 on LT. International LT current page (I1) omits A26. The 2017 release (I2) lists A26 in LT. Current status not confirmed.

# 7. Unverified combinations (user-entered path applies, with label)

Volvo: VNL 440, 640, 660, 840 (all engines). VNL 300, 400, 740, 760, 860 with D11, D16, Cummins X15, X12G, D13TC (interval level).

Freightliner Cascadia: DD16 (option), X15N (interval), prior-generation Cascadia (interval).

Peterbilt 579: X15N, ISX12N, L9N (all), 579EV.

Kenworth: X15N (interval), MX-11 and X15 diesel (option).

Mack: Anthem MP7, MP8, MP8 HE, MP10 (option and interval). MP13 and MP13 HE (per-engine interval). Granite, Pinnacle, TerraPro (all).

International: S13 (interval), A26 (interval) and current option on LT, LoneStar and HX (all).

# 8. Sources

| ID | Document | URL | Checked |
| --- | --- | --- | --- |
| V1 | Volvo VNL specifications page (VNL 300, 400, 740, 760, 860; all D13) | https://www.volvotrucks.us/trucks/vnl/specifications/ | 2026-10-18 |
| V2 | Volvo VNL Product Guide, PV835-1190, 01/2024 (D13 ratings, p. 41) | https://www.volvotrucks.us/trucks/all-new-vnl/Files/volvo-vnl-product-guide-brochure.pdf | 2026-10-18 |
| V3 | Volvo VNL brochure 2021, PV835-1139 (D11, D13, D13TC, X12G, X15; pp. 23-26, 38, 69-70) | https://www.volvotrucks.us/media/vtna/files/shared/trucks/brochures/5395-volvo-vnl-brochure-2021-digital-5.pdf | 2026-10-18 |
| V4 | Volvo SAAG poster, Manufactured Year 2022 GHG and Newer (D11/D13 pan; cited in the earlier report) | https://www.volvotrucks.us/media/vtna/files/saag-2022-ghg-newer-poster-011023-eng.pdf | 2026-10-18 |
| V5 | Volvo SAAG poster, Manufactured Year 2021 GHG and Newer (Batch 1, section 3.2) | https://www.volvotrucks.us/media/vtna/files/shared/parts-and-services/maintenance-intervals/2021/saag-2021-ghg-newer-poster-062221-eng.pdf | 2026-10-18 |
| F1 | Freightliner release: fifth-generation Cascadia, October 15, 2024 (DD13, DD15, X15, X15N) | https://northamerica.daimlertruck.com/news-stories/2024/freightliner-unveils-fifth-generation-cascadia | 2026-10-18 |
| F2 | Detroit Cascadia engine ratings page (DD13 and DD15 at 400 hp; about 2016) | https://dtnacontent-dtna.prd.freightliner.com/content/dam/enterprise/documents/87338_DET_Cascadia_Product_Overview_WEB.pdf | 2026-10-18 |
| S1 | Detroit DD Platform Gen 5 Operator Manual, DDC-SVC-MAN-0217 (Batch 1) | https://dtnacontent-dtna.prd.freightliner.com/content/dam/public/dtna-servicelit/ddc/pdfs/OperatorsManual/DDPlatform/DDC-SVC-MAN-0217_2026.pdf | 2026-10-18 |
| S2 | Detroit Lubricating Oil, Fuel, and Filters, DDC-SVC-BRO-0001, 2021 (Batch 1) | https://dtnacontent-dtna.prd.freightliner.com/content/dam/public/dtna-servicelit/ddc/pdfs/Lube_Oil_Coolant/DDC-SVC-BRO-0001_2021.pdf | 2026-10-18 |
| P1 | Peterbilt Model 579 page (engine list with horsepower) | https://www.peterbilt.com/trucks/on-highway/579 | 2026-10-18 |
| K1 | Kenworth release: PACCAR MX-13 CARB-compliant engine for T680, T880, W990 | https://www.kenworth.com/about-us/news/kenworth-introduces-paccar-mx-13-carb-compliant-engine-for-t680-t880-and-w990-models/ | 2026-10-18 |
| K2 | Kenworth release: production schedule, trucks specified with Cummins X15N | https://www.kenworth.com/about-us/news/kenworth-announces-production-schedule-for-class-8-trucks-specified-with-new-cummins-x15n-engine/ | 2026-10-18 |
| M1 | Mack article: 2026 Anthem powertrain upgrades, dated 11/24/2025 | https://www.macktrucks.com/magazine/articles/2025/november/mack-anthem-powertrain-upgrades | 2026-10-18 |
| M2 | Mack engines page (MP13, MP13 HE, MP7, MP8, MP8 HE) | https://www.macktrucks.com/powertrain/engines/ | 2026-10-18 |
| I1 | International LT series page (S13 and Cummins X15) | https://www.international.com/products/trucks/series/lt | 2026-10-18 |
| I2 | International release: first LT and RH shipments with A26, June 30, 2017 | https://news.international.com/2017-06-30-International-Truck-Ships-First-On-Highway-Vehicles-With-A26-Engine | 2026-10-18 |
| I3 | International A26 Change Oil Service Interval, A26_SI_03172022 (Batch 1 reference) | https://www.international.com/-/media/Project/TratonGroup/International/International/Vehicles/Resources/Tem-Body-Builder/Engine-Media/EPA-21-International-A26/A26_Service_Interval_PDF.pdf?rev=68969439a1c8401cb4cdce781a375e0e | 2026-10-18 |
| C1 | Cummins X15 Quick Reference Guide, EPA 2024, Bulletin 6553662, Rev 11/24 | https://mart.cummins.com/imagelibrary/data/assetfiles/0077777.pdf | 2026-10-18 |
| C2 | Cummins X15 Quick Reference Guide, EPA 2017, Bulletin 5410641, Rev 11/17 | https://mart.cummins.com/imagelibrary/data/assetfiles/0032808.pdf | 2026-10-18 |
| E108 | PACCAR ESB E108, rev 1/19/2015 (earlier report) | https://partsandservice.peterbilt.com/img/pb/guides/MX_Maintenance_Recommendations_Paccar_MX_Engine.pdf | 2026-10-18 |
| E256 | PACCAR SB E256, released 1/6/2020 (earlier report; NHTSA copy) | https://static.nhtsa.gov/odi/tsbs/2020/MC-10171027-0001.pdf | 2026-10-18 |